# SIMATSENGINEERING

# ASSESSMENTTOOL 1

COURSENAME:COMPUTERNETWORKS COURSECODE: CSA0708

COURSEOUTCOMECOVERED

CO2:ConfigureIPaddressingschemes, subnetting,andbasicroutingtechniques using simulation tool, for efficient network design and topology. (BL3)

AssessmentTool Weightage: 40%

QUESTIONS:5	TotalMarks:50

AnnexureA

# ANALYTICALPROBLEMSOLVING

CodeofConduct:

I  S DHANUSH (192524317) certifythatthissubmissionismyoriginalworkandthatIhave adhered to the guidelines specified for this assessment.

Iunderstandthatanyviolationofacademicintegrityruleswillresultindisciplinary action.

Signatureofthestudent:

S DHANUSH (192524317)

AnnexureA

# ANALYTICALPROBLEMSOLVING

- Auniversity network is assigned the IP address block 192.168.20.0/24 for its internal communication. The network administrator has decided to divide this network into subnets for three different labs using fixed subnet masks. LabAis assigneda/26subnet,LabBisassigneda/27subnet,andLabCisassigneda/28 subnet.Basedonthegivensubnetmasks,applysubnettingconceptstodetermine the network address of each lab. Further, identify the valid range of host IP addresses and the broadcast address for each subnet. Finally, calculate the total number of usable host IPaddresses available in each lab’s subnet.

- Anetworkadministratorhasconfiguredtwosystemswiththefollowingdetails: System 1 has IP address 192.168.1.130 with subnet mask 255.255.255.192, and System2hasIPaddress192.168.1.190withsubnetmask255.255.255.192.Using the given configuration, apply subnetting techniques to determine the subnet range,network address,andbroadcastaddress foreachsystem.Identifythevalid hostIPrangeforbothsubnetsbasedonthegivensubnetmask.Calculatewhether each IP address falls within its respective subnet range. Also, determine the number of usable host addresses available in each subnet.

- An organization has been allocated the IP block 172.16.0.0/16 for its internal network. The network administrator has already decided the subnet masks for eachdepartmentasfollows:HR→/25,Sales→/26,IT→/27,andAdmin→

/28. Using the given subnet masks, apply subnetting techniques to assign suitablesubnetaddressesforeachdepartment.Determinethenetworkaddress, valid host IP range, and broadcast address for each subnet. Based on the allocation, calculate thenumber ofusablehostaddresses in each department.

Finally,identifytheremainingunusedIPaddressrangewithinthegiven network block.

- AcompanyusestheIPaddress172.16.10.0/24andhasimplementedsubnetting

usinga/27subnetmaskforitsinternaldepartments.Usingthegivensubnet mask, apply subnetting techniques to calculate the total number of subnets created and the number of usable hosts per subnet. Determine the subnet to whichtheIPaddress172.16.10.78belongs.Identifythenetworkaddressand broadcast address for that subnet. Also, determine whether the IP address 172.16.10.95 falls within the same subnet range based on the given configuration.

- A network uses the Internet Protocol version 6 address 2001:0db8:0000:0000:0000:ff00:0042:8329.Apply IPv6 addressing rules to compress the given address into its shortest form and then expand the compressed address back to its full notation. Using a prefix length of /64, determine the network prefix of the given address.Also, calculate the total number of possible host addresses available within this network based on the given prefix length.

Subnetting192.168.20.0/24forThreeLabs

Given:

  - Network:192.168.20.0/24

  - LabA:/26

  - LabB:/27

  - LabC:/28 Lab A — /26 Subnet mask:

/26 = 255.255.255.192

Numberof addresses:

Usable hosts:

Startingfrom 192.168.20.0:

232−26=26=64

64−2=62

| Item | Address |
| --- | --- |
| Networkaddress | 192.168.20.0 |
| First host | 192.168.20.1 |
| Lasthost | 192.168.20.62 |
| Broadcast | 192.168.20.63 |
| Usable hosts | 62 |

LabB— /27

ThenextavailableaddressafterLabAis192.168.20.64. Subnet mask:

/27 = 255.255.255.224

Numberof addresses:

Usable hosts:

232−27=25=32

32−2=30

| Item | Address |
| --- | --- |
| Networkaddress | 192.168.20.64 |
| First host | 192.168.20.65 |
| Lasthost | 192.168.20.94 |
| Broadcast | 192.168.20.95 |
| Usable hosts | 30 |

LabC— /28

Thenextavailableaddressis192.168.20.96. Subnet mask:

/28 = 255.255.255.240

Numberof addresses:

Usable hosts:

232−28=24=16

16−2=14

| Item | Address |
| --- | --- |
| Networkaddress | 192.168.20.96 |
| First host | 192.168.20.97 |
| Lasthost | 192.168.20.110 |
| Broadcast | 192.168.20.111 |
| Usable hosts | 14 |

FinalAnswer—Q1

| Lab | Prefix | Network | ValidHost Range | Broadcast | Usable Hosts |
| --- | --- | --- | --- | --- | --- |
| LabA | /26 | 192.168.20.0 | .1 – .62 | 192.168.20.63 | 62 |
| LabB | /27 | 192.168.20.64 | .65 – .94 | 192.168.20.95 | 30 |
| LabC | /28 | 192.168.20.96 | .97 – .110 | 192.168.20.111 | 14 |

Important: Since these are fixed-size subnets with different masks, this allocation is effectivelyusingdifferent-sizedsubnetblockswithinthe/24.Strictlyspeaking,thisisVLSM (VariableLengthSubnetMasking)ratherthanFLSM,because/26,/27and/28aredifferent masks.

Systems1and2 —192.168.1.130and 192.168.1.190

Given:

  - Subnetmask=255.255.255.192

  - Prefix = /26 Step1:Findblocksize

256−192=64

Therefore,the/26subnetsare:

  - 192.168.1.0 – 63

  - 192.168.1.64 – 127

  - 192.168.1.128 – 191

  - 192.168.1.192 – 255

Both 130 and 190 lie between 128 and 191. Therefore,bothsystemsbelongtothesamesubnet. System 1 — 192.168.1.130

| Item | Address |
| --- | --- |
| IPaddress | 192.168.1.130 |
| Networkaddress | 192.168.1.128 |
| First host | 192.168.1.129 |
| Lasthost | 192.168.1.190 |
| Broadcast | 192.168.1.191 |

192.168.1.130isavalidhostaddress. System 2 — 192.168.1.190

| Item | Address |
| --- | --- |
| IPaddress | 192.168.1.190 |
| Networkaddress | 192.168.1.128 |
| First host | 192.168.1.129 |
| Lasthost | 192.168.1.190 |
| Broadcast | 192.168.1.191 |

192.168.1.190isalsoavalidhostaddress. Number of usable hosts

For /26:

232−26=64

64−2=62

FinalAnswer—Q2

| System | IP | Network | ValidHost Range | Broadcast | Valid? |
| --- | --- | --- | --- | --- | --- |
| System 1 | 192.168.1.130 | 192.168.1.128 | .129 – .190 | .191 | Yes |
| System 2 | 192.168.1.190 | 192.168.1.128 | .129 – .190 | .191 | Yes |

Bothsystemsareinthesamesubnet. Usable hosts = 62

- DepartmentSubnetting— 172.16.0.0/16

Givennetwork:

172.16.0.0/16

Departments:

  - HR → /25

  - Sales→/26

  - IT→ /27

  - Admin → /28

Becausethesubnetsizesaredifferent,thisisVLSM. We assign the largest subnet first.

HR—/25

Numberof addresses:

Usable:

Assign:

172.16.0.0/25

232−25=128

128−2=126

| Item | Address |
| --- | --- |
| Network | 172.16.0.0 |
| First host | 172.16.0.1 |
| Lasthost | 172.16.0.126 |

| Broadcast | 172.16.0.127 |
| --- | --- |
| Usable hosts | 126 |

Sales—/26

Thenextavailableaddressis172.16.0.128. Number of addresses:

Usable:

26=64

64−2=62

| Item | Address |
| --- | --- |
| Network | 172.16.0.128 |
| First host | 172.16.0.129 |
| Lasthost | 172.16.0.190 |
| Broadcast | 172.16.0.191 |
| Usable hosts | 62 |

IT—/27

Nextavailableaddress:

172.16.0.192

Numberof addresses:

Usable:

25=32

32−2=30

| b | Address |
| --- | --- |
| Network | 172.16.0.192 |
| First host | 172.16.0.193 |
| Lasthost | 172.16.0.222 |
| Broadcast | 172.16.0.223 |
| Usable hosts | 30 |

Admin—/28

Nextavailableaddress:

172.16.0.224

Numberof addresses:

Usable:

24=16

16−2=14

| Item | Address |
| --- | --- |
| Network | 172.16.0.224 |
| First host | 172.16.0.225 |
| Lasthost | 172.16.0.238 |
| Broadcast | 172.16.0.239 |
| Usable hosts | 14 |

FinalAllocation— Q3

| Department | Prefix | Network | Host Range | Broadcast | Usable Hosts |
| --- | --- | --- | --- | --- | --- |
| HR | /25 | 172.16.0.0 | .1 – .126 | .127 | 126 |
| Sales | /26 | 172.16.0.128 | .129 – .190 | .191 | 62 |
| IT | /27 | 172.16.0.192 | .193 – .222 | .223 | 30 |
| Admin | /28 | 172.16.0.224 | .225 – .238 | .239 | 14 |

Remainingunusedrange

Theoriginal block is:

172.16.0.0– 172.16.255.255

Allocatedsofar:

172.16.0.0– 172.16.0.239

Therefore,theremainingunusedaddress rangeis:

Thisincludestheremainingaddressesinthe/24 portionandalloftheremaining/16 block.

- 172.16.10.0/24SubnettedUsing/27

Given:

  - Originalnetwork:172.16.10.0/24

  - Newsubnetmask: /27

  - Mask:255.255.255.224

Step1:Numberofsubnets

Originalprefix=/24 New prefix = /27

Borrowedbits:

Numberof subnets:

Step2:Usablehostspersubnet

Host bits:

Totaladdresses:

Usable:

27−24=3

23=8

32−27=5

25=32

256−224=32

Thesubnetsare:

| Subnet | Range | Broadcast |
| --- | --- | --- |
| 1 | .0 – .31 | .31 |
| 2 | .32 – .63 | .63 |
| 3 | .64 – .95 | .95 |
| 4 | .96 – .127 | .127 |
| 5 | .128 – .159 | .159 |
| 6 | .160 – .191 | .191 |
| 7 | .192 – .223 | .223 |
| 8 | .224 – .255 | .255 |

TheIP172.16.10.78liesbetween64and95. Therefore:

Network address = 172.16.10.64 Broadcastaddress=172.16.10.95

Validhostrange=172.16.10.65–172.16.10.94

So:

172.16.10.78belongsto172.16.10.64/27

What about 172.16.10.95? 172.16.10.95isthebroadcastaddress.

Therefore,itbelongstothesubnetrange,butitcannotbeassignedtoanormalhost. Final Answer — Q4

  - Numberofsubnets= 8

  - Usablehosts/subnet=30

  - 172.16.10.78 → 172.16.10.64/27

  - Networkaddress=172.16.10.64

  - Validhostrange=172.16.10.65–172.16.10.94

  - Broadcast=172.16.10.95

  - 172.16.10.95→Yes,itiswithinthesubnet’saddressrange,butitisthebroadcast address, so it is not a valid host IP.

- Ipv6AddressCompressionandExpansion Given Ipv6 address: 2001:0db8:0000:0000:0000:ff00:0042:8329

Step1:Removeleadingzeros

Ipv6allowsleadingzerosineachhexadecimalgrouptoberemoved. Therefore:

2001:db8:0:0:0:ff00:42:8329

Therearethreeconsecutive0 groups:

0:0:0

These can be replaced by ::. Therefore,theshortestformis:

2001:db8::ff00:42:8329

CompressedIpv6address 2001:db8::ff00:42:8329

Step2:Expandthecompressedaddress

Compressed:

2001:db8::ff00:42:8329

AnIpv6addressmustcontain8groups. Currently we have:

1.2001

- db8

- ff00

- 42

- 8329

So :: represents 3 groups of 0000.Therefore: 2001:0db8:0000:0000:0000:ff00:0042:8329

Sotheexpandedform is:

2001:0db8:0000:0000:0000:ff00:0042:8329

Step 3: Determine the /64 network prefix Given: 2001:0db8:0000:0000:0000:ff00:0042:8329/64

AnIpv6addresshas128 bits.

With/64,thefirst64bitsarethenetworkportion. Each hexadecimal group contains 16 bits:

64/16=4

Therefore,the firstfourgroupsformthenetworkprefix:

2001:0db8:0000:0000

Thenetworkprefix incompressedform is:

2001:db8::/64

Step4:Numberofpossiblehostaddresses

Ipv6 has128 bits. Networkbits=64 Host bits:

128−64=64

Numberofpossible hostaddresses:

264

18,446,744,073,709,551,616

UnliketraditionalIpv4subnetcalculations,Ipv6generallydoesnotsubtractnetworkand broadcast addresses in the same way; Ipv6 does not use a broadcast address.

RUBRICSFORCALCULATION

| Criteria | Weightag e | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Problem Understandin g | 20% | Clearly interprets problem, identifiesall parameters and requirements accurately | Understands problemwith minorgaps | Partial understanding ;misseskey elements | Misinterprets orunableto understand problem |
| Method Selection | 20% | Selectsmost appropriate method/form u la with strong justification | Selects correct methodwith minorerrors | Inappropriate orpartially correct method | Unableto select method |
| Solution Process | 25% | Logical, systemati c steps with correct approach throughout | Mostly correctsteps withminor mistakes | Disorganized steps; multiple errors | Noclear process |
| Accuracyof Calculation | 20% | All calculation saccurate; correct final answer | Minor calculation errors | Frequent errors; incorrectfinal answer | Novalid calculations |
| Interpretation ofResults | 15% | Clearly interprets results with units and engineerin g relevance | Basic interpretatio nwithminor omissions | Limitedor unclear interpretation | No interpretatio n |