# SIMATSENGINEERING

ASSESSMENTTOOL-2

# COURSENAME:COMPUTERNETWORKS COURSE CODE: CSA0708

# COURSEOUTCOMECOVERED

CO2:ConfigureIPaddressingschemes, subnetting,andbasicroutingtechniques using simulation tool, for efficient network design and topology. (BL3)

AssessmentTool Weightage: 30%

# QUESTIONS:5	TotalMarks:50

AnnexureA

Scenario-Based

CodeofConduct:

I S DHANUSH (192524317) certifythatthissubmissionismyoriginalworkandthatIhave adhered to the guidelines specified for this assessment.

Iunderstandthatanyviolationofacademicintegrityruleswillresultindisciplinary action.

Signatureofthestudent:

S DHANUSH (192524317)

# SubnetAllocationforSmart Campus

GivenIPblock:192.168.100.0/24

A/24network contains:

  - Totaladdresses=232−24=256

  - Addressrange=192.168.100.0 – 192.168.100.255

Thedepartmentsare assigned/25,/26,and/27.

# SchoolofComputing—/25

Subnetmask:255.255.255.128

  - Blocksize=128 addresses

  - Networkaddress=192.168.100.0

  - Broadcastaddress=192.168.100.127

  - Validhostrange=192.168.100.1–192.168.100.126

  - Usablehosts= 128−2=126

# SchoolofElectronics—/26

Thenextavailableblockstartsat192.168.100.128. Subnet mask: 255.255.255.192

  - Blocksize=64 addresses

  - Networkaddress=192.168.100.128

  - Broadcastaddress=192.168.100.191

  - Validhostrange=192.168.100.129–192.168.100.190

  - Usablehosts= 64−2=62

# SchoolofMechanicalEngineering—/27

Thenextavailableblockstartsat192.168.100.192. Subnet mask: 255.255.255.224

  - Blocksize=32 addresses

  - Networkaddress=192.168.100.192

  - Broadcastaddress=192.168.100.223

  - Validhostrange=192.168.100.193–192.168.100.222

  - Usablehosts= 32−2=30

# RemainingUnusedIPAddressRange

The/24blockendsat 192.168.100.255.

TheMechanicalEngineeringsubnetendsat192.168.100.223. Therefore, the remaining unused addresses are:

192.168.100.224– 192.168.100.255

Thisgives32totaladdressesremaining.

Ifthisremaining blockistobeusedasanothersubnet, itcan beallocated as:

  - Networkaddress: 192.168.100.224

  - Broadcastaddress:192.168.100.255

  - Validhostrange:192.168.100.225–192.168.100.254

  - Usablehosts: 30

  - CIDR:/27

AddressAllocationDiagram

Answer:Thethreedepartmentsrequire218usablehostaddressesintotal(126+62+ 30), and the remaining /27 block from 192.168.100.224/27 is available for future expansion.

# SubnetAllocationfortheSoftwareCompany

GivenIPblock:10.10.0.0/16

The/16 block provides:

  - Totaladdresses=216=65,536

  - Overallrange=10.10.0.0 – 10.10.255.255

Thesubnetsareallocatedsequentially.

# SoftwareDevelopment—/24

A/24subnetcontains 256 addresses.

  - NetworkAddress:10.10.0.0

  - BroadcastAddress:10.10.0.255

  - ValidHostRange:10.10.0.1–10.10.0.254

  - UsableHosts:256−2=254

SubnetMask:255.255.255.0

# QualityAssurance—/25

Thenextavailableaddressis10.10.1.0. A /25 subnet contains 128 addresses.

  - NetworkAddress:10.10.1.0

  - BroadcastAddress:10.10.1.127

  - ValidHostRange:10.10.1.1–10.10.1.126

  - UsableHosts:128−2=126

SubnetMask:255.255.255.128

# HumanResources—/26

Thenextavailableaddressis10.10.1.128. A /26 subnet contains 64 addresses.

  - NetworkAddress:10.10.1.128

  - BroadcastAddress:10.10.1.191

  - ValidHostRange:10.10.1.129–10.10.1.190

  - UsableHosts:64−2=62

SubnetMask:255.255.255.192

# ExecutiveManagement —/27

Thenextavailableaddressis10.10.1.192. A /27 subnet contains 32 addresses.

  - NetworkAddress:10.10.1.192

  - BroadcastAddress:10.10.1.223

  - ValidHostRange:10.10.1.193–10.10.1.222

  - UsableHosts: 32-2=30

SubnetMask:255.255.255.224

# UnusedIPAddressRange

TheExecutiveManagementsubnetendsat:

10.10.1.223

Theoriginal /16 block endsat:

10.10.255.255

Therefore,theremainingunusedIPaddressrangeis:

10.10.1.224– 10.10.255.255

Thisrepresents65,056unusedaddresses. The first unused portion is:

10.10.1.224– 10.10.1.255

followedbytheentirerange:

10.10.2.0– 10.10.255.255

AddressAllocationDiagram:

# Totalusablehostsallocated:

254 +126 +62+30=472hosts

# Totalunusedaddresses:65,056addresses.

# IPv6 Subnet Calculation – Smart Hospital Given IPv6 network: 2001:0db8:abcd:0000:0000:0000:0000:0000/64

AnIPv6addresshas128 bits,dividedintoeight16-bithexadecimalgroups.

# CompressedNotation

Given: 2001:0db8:abcd:0000:0000:0000:0000:0000/64

Theconsecutivegroupsofzeroscanbereplacedby::. Therefore:

# CompressedIPv6Address

2001:db8:abcd::/64

- ExpandtheCompressedAddress Compressed address: 2001:db8:abcd::/64

Therearethreeexplicitlywrittengroups: 2001 : db8 : abcd

The::representstheremainingfivegroupsofzeros. Therefore, the full hexadecimal representation is: 2001:0db8:abcd:0000:0000:0000:0000:0000/64

# IdentifytheNetwork Prefix

Theprefixlengthis/64. This means:

  - First64bits=Networkportion

  - Remaining 64 bits = Interface/host portion Thefirstfour16-bitgroupsformthenetworkprefix: 2001:0db8:abcd:0000

Therefore,thenetworkprefixis:

2001:0db8:abcd:0000/64 or, in compressed form: 2001:db8:abcd::/64

# Calculatethe Numberof HostAddresses

IPv6has128bits. Given prefix:

/64

Host bits:

Therefore:

128−64=64

264

Sothenumberofpossiblehostaddressesis:

18,446,744,073,709,551,616

# TotalPossibleHostAddresses

The/64 subnet contains:

# 18,446,744,073,709,551,616 possible addresses

UnlikeIPv4,IPv6doesnotsubtracttwoaddressesfornetworkandbroadcastaddresses in the usual subnet calculation, because IPv6 has no broadcast address.

FinalAnswer

| Requirement | Answer |
| --- | --- |
| GivenIPv6 | 2001:0db8:abcd:0000:0000:0000:0000:0000/64 |
| Compressed notation | 2001:db8:abcd::/64 |
| Expandednotation | 2001:0db8:abcd:0000:0000:0000:0000:0000/64 |
| Networkprefix | 2001:0db8:abcd:0000/64 |
| Networkbits | 64 bits |
| Host/interfacebits | 64 bits |
| Totaladdresses | 2⁶⁴ |
| Totalpossibleaddresses | 18,446,744,073,709,551,616 |

Simple Diagram:

# RoutingTableLookup

Givenroutingtable:

192.168.1.0/24

192.168.2.0/24

192.168.3.0/24

DestinationIP:

192.168.2.45

# DestinationSubnet

The/24maskmeansthefirstthreeoctetsidentifythenetwork. Destination IP:192.168.2.45

Subnetmask:	255.255.255.0

─────────────

Network:	192.168.2.0

Therefore,thedestinationbelongsto:

192.168.2.0/24

# MatchingRoutingTableEntry

Comparethedestinationwith therouting table:

| RoutingEntry | Match? |
| --- | --- |
| 192.168.1.0/24 | +No |
| 192.168.2.0/24 | ⬛Yes |
| 192.168.3.0/24 | +No |

Sothe matchingentry is:

192.168.2.0/24

# Next-HopNetwork/Interface

Therouter forwardsthepackettowardtheinterfaceconnectedto:

192.168.2.0/24

Theexactnext-hopIP/interfaceisnotprovidedinthequestion,soitcannotbe determined numerically.

For example:

Destination: 192.168.2.45

↓

Routerchecksroutingtable

↓

Matches192.168.2.0/24

↓

Forwardthroughinterfaceconnected to 192.168.2.0/24

# Default Route

Ifthedestinationdoesnotmatchanyspecificrouting-tableentry,therouterusesa default route:

0.0.0.0/0

Thedefaultroutemeans anyIPv4destinationthathasnomore-specificmatching route.

FinalAnswers

| Question | Answer |
| --- | --- |
| DestinationIP | 192.168.2.45 |
| Destination subnet | 192.168.2.0/24 |
| Matchingentry | 192.168.2.0/24 |

| Next-hop/interface | Interfaceconnectedto 192.168.2.0/24 |
| --- | --- |
| Defaultroute | 0.0.0.0/0 |

Simple Routing Diagram

Conclusion:Since192.168.2.45fallswithin192.168.2.0/24,therouterselectsthat routing-table entry and forwards the packet through the interface connected to that subnet. If no route matches, it uses 0.0.0.0/0.

# Layer-3Router–LANIPAddressing

Given:

  - AdministrationLAN:192.168.10.0/26

  - FinanceLAN:192.168.10.64/26

  - AdministrationGateway:192.168.10.1

  - FinanceGateway:192.168.10.65

A/26subnethas64totaladdressesand62usablehostaddresses.

- Administration LAN Network:192.168.10.0/26

  - Networkaddress: 192.168.10.0

  - Broadcastaddress:192.168.10.63

  - Validhostrange:192.168.10.1–192.168.10.62

  - Defaultgateway:192.168.10.1

# ThreesuitablecomputerIPs

| Computer | IPAddress | DefaultGateway |
| --- | --- | --- |
| Computer1 | 192.168.10.2 | 192.168.10.1 |
| Computer2 | 192.168.10.3 | 192.168.10.1 |
| Computer3 | 192.168.10.4 | 192.168.10.1 |

# Finance LAN

Network:192.168.10.64/26

  - Networkaddress: 192.168.10.64

  - Broadcastaddress:192.168.10.127

  - Validhostrange:192.168.10.65–192.168.10.126

  - Defaultgateway:192.168.10.65

Threesuitable computer IPs

| Computer | IPAddress | DefaultGateway |
| --- | --- | --- |
| Computer1 | 192.168.10.66 | 192.168.10.65 |
| Computer2 | 192.168.10.67 | 192.168.10.65 |
| Computer3 | 192.168.10.68 | 192.168.10.65 |

FinalAnswer

| LAN | Net wor k Add ress | Valid Host Rang e | Broa dcast | Gat ewa y |
| --- | --- | --- | --- | --- |
| Admini stratio n | 192.1 68.10 .0 | 192.16 8.10.1 – 192.16 8.10.6 2 | 192.16 8.10.6 3 | 192.1 68.10 .1 |
| Financ e | 192.1 68.10 .64 | 192.16 8.10.6 5– 192.16 8.10.1 26 | 192.16 8.10.1 27 | 192.1 68.10 .65 |

NetworkDiagram:

Keypoint:HostsintheAdministrationLANuse192.168.10.1astheirdefaultgateway, while hosts in the Finance LAN use 192.168.10.65.