# SIMATSENGINEERING

ASSESSMENTTOOL-3

# COURSENAME:COMPUTERNETWORKS COURSE CODE: CSA0708

# COURSEOUTCOMECOVERED

CO2:ConfigureIPaddressingschemes,subnetting,andbasicrouting techniques using simulation tool, for efficient network design and topology. (BL3)

AssessmentTool Weightage: 30%

# QUESTIONS:5	TotalMarks:50

AnnexureA

Debugging&OptimisationTask

CodeofConduct:

I S DHANUSH (192524317)certifythatthissubmissionismyoriginalworkandthatI have adhered to the guidelines specified for this assessment.

Iunderstandthatanyviolationofacademicintegrityruleswillresultindisciplinaryaction.

Signatureofthestudent:

S DHANUSH (192524317)

# 1).Given Configuration

- IPAddress:192.168.10.65

- SubnetMask:255.255.255.192 (/26)

- DefaultGateway:192.168.10.1

- RequiredSubnet:192.168.10.0/26

# ​IdentifytheSubnet

For/26,eachsubnetcontains64addresses. The subnets are:

| Subnet | NetworkAddress | BroadcastAddress | ValidHostRange |
| --- | --- | --- | --- |
| 1 | 192.168.10.0 | 192.168.10.63 | 192.168.10.1 – 192.168.10.62 |
| 2 | 192.168.10.64 | 192.168.10.127 | 192.168.10.65 – 192.168.10.126 |

Therefore,192.168.10.65belongstothe192.168.10.64/26subnet,not192.168.10.0/26.

# ​Network Details

FortheconfiguredIP192.168.10.65/26:

  - NetworkAddress:192.168.10.64

  - BroadcastAddress:192.168.10.127

  - ValidHostRange:192.168.10.65–192.168.10.126

# UsableHosts: 62

- Verify IPand Gateway IPAddress:192.168.10.65

⬛Validhostaddress forthe192.168.10.64/26subnet.

# Gateway:192.168.10.1

+Invalidforthissubnetbecauseitbelongsto192.168.10.0/26.

Theworkstationanddefaultgatewaymustnormallybeinthesamesubnetforthisbasic configuration.

# ​ConfigurationErrors

ThemainerroristhemismatchbetweentheIPaddressandgateway.

  - Workstation:192.168.10.65/26→subnet192.168.10.64/26

  - Gateway:192.168.10.1→subnet192.168.10.0/26

Therefore,theworkstationcannot properlyuse 192.168.10.1asitsgateway.

# ​CorrectedConfiguration

Asuitableconfigurationis:

| Setting | CorrectValue |
| --- | --- |
| IPAddress | 192.168.10.66 |
| SubnetMask | 255.255.255.192 (/26) |
| DefaultGateway | 192.168.10.65 |
| NetworkAddress | 192.168.10.64 |
| BroadcastAddress | 192.168.10.127 |

Here,boththe workstationandgatewayareinthe192.168.10.64/26subnet.

Note:Thegatewaycouldbeanyvalidhostaddressinthissubnet,providedthataddressis actually configured on the router. For example, 192.168.10.65.

# TotalUsableHostAddresses

For/26:

  - Totaladdresses=2⁶=64

  - Networkaddress=1

  - Broadcastaddress=1

Usablehostaddresses=64−2=62 FinalAnswer

The IP 192.168.10.65 belongs to the 192.168.10.64/26 subnet. Its valid host range is 192.168.10.65–192.168.10.126, with network address 192.168.10.64 and broadcast address 192.168.10.127.TheconfiguredIPisvalid,butthegateway192.168.10.1isincorrectbecause it belongs to another subnet.Acorrected configuration can use IP192.168.10.66 and gateway 192.168.10.65. The subnet provides 62 usable host addresses.

# ​IPv6AddressingDebugging Given Configuration

  - DeviceA:2001:db8::ff00:42:8329/64

  - DeviceB:2001:db8:0:0:1::1/64

- Expand the IPv6 Addresses DeviceA: 2001:db8:0:0:0:ff00:42:8329/64 DeviceB:2001:db8:0:0:1:0:0:1/64

# DeterminetheNetwork Prefix

Sincetheprefixis/64,thefirst4groups(64bits)representthe network.

| Device | IPv6Address | NetworkPrefix |
| --- | --- | --- |
| A | 2001:db8:0:0:0:ff00:42:8329 | 2001:db8:0:0::/64 |
| B | 2001:db8:0:0:1:0:0:1 | 2001:db8:0:0:1::/64 |

# CheckWhetherTheyAreintheSameSubnet

  - DeviceA→ 2001:db8:0:0::/64

  - DeviceB→ 2001:db8:0:0:1::/64

# +Theyarenotinthesame/64subnet.

Thedifferenceisinthe5thhexadecimalgroup:

  - DeviceA:0

  - DeviceB:1

Therefore,communicationfailsiftheyareexpectedtocommunicatedirectlyonthesame IPv6 subnet.

# ConfigurationError

Theproblemisanetworkprefixmismatch. Device A is configured in:

# 2001:db8:0:0::/64

DeviceBisconfiguredin:

# 2001:db8:0:0:1::/64

Theyshouldhavethesamefirst 64bitsiftheyareonthesameLayer-2IPv6 network.

# CorrectedConfiguration

KeepDeviceAunchangedand assignDeviceBanaddressfromthesame subnet.

# DeviceA:

2001:db8:0:0:ff00:42:8329/64

# DeviceB:

2001:db8:0:0::2/64

Bothnowbelongto:

# Network:2001:db8:0:0::/64

Socommunicationcanworkonthesamesubnet,assumingthelinkandotherIPv6settings are correctly configured.

# NumberofAvailableHostAddresses

Foran IPv6/64 subnet:

  - Totalhostbits=128−64=64bits

  - Numberofaddresses =2⁶⁴

Availableaddresses=18,446,744,073,709,551,616

UnlikeIPv4,IPv6doesnotnormallysubtractnetworkandbroadcastaddresses,becauseIPv6 has no broadcast address.

# FinalAnswer

DeviceAbelongs to 2001:db8:0:0::/64, while Device B belongs to 2001:db8:0:0:1::/64. Therefore, they are in different subnets. The prefix mismatch is the configuration error. AssigningDeviceBanaddresssuchas2001:db8:0:0::2/64placesbothdevicesin thesame subnet.A /64 IPv6 subnet contains 2⁶⁴ = 18,446,744,073,709,551,616 addresses.

# ​SubnettingandVLSMOptimization Given Network

Network:192.168.1.0/24 A /24 network contains:

  - TotalIPaddresses=2⁸=256

  - Networkaddress=192.168.1.0

  - Broadcastaddress= 192.168.1.255

  - Usablehostaddresses= 254

Theorganizationcurrentlyuses/26,/27,and/28subnetmasks.

# AnalyzetheExisting Subnets

  - ​/26 Subnet

A/26provides64totaladdressesand62usablehostaddresses.

| Item | Address |
| --- | --- |
| NetworkAddress | 192.168.1.0 |
| First Host | 192.168.1.1 |
| LastHost | 192.168.1.62 |

| Broadcast | 192.168.1.63 |
| --- | --- |
| Usable Hosts | 62 |

# ​/27 Subnet

Thenextavailableblock startsat 192.168.1.64.

A/27provides32totaladdressesand30usablehosts.

| Item | Address |
| --- | --- |
| NetworkAddress | 192.168.1.64 |
| First Host | 192.168.1.65 |
| LastHost | 192.168.1.94 |
| Broadcast | 192.168.1.95 |
| Usable Hosts | 30 |

# ​/28 Subnet

Thenextavailableblock startsat 192.168.1.96.

A/28provides16totaladdressesand14usablehosts.

| Item | Address |
| --- | --- |
| NetworkAddress | 192.168.1.96 |
| First Host | 192.168.1.97 |
| LastHost | 192.168.1.110 |
| Broadcast | 192.168.1.111 |
| Usable Hosts | 14 |

# ExistingAddressUtilization

The important point is that the question does not specify the actual number of devices requiredbyeachdepartment.Therefore,exactunusedaddressescannotbecalculatedfrom the given information alone.

Forexample,ifthedepartmentsactuallyneedthe following:

| Department | CurrentMask | Usable IPs | Required Hosts | UnusedIPs |
| --- | --- | --- | --- | --- |
| DepartmentA | /26 | 62 | 50 | 12 |
| DepartmentB | /27 | 30 | 15 | 15 |
| DepartmentC | /28 | 14 | 5 | 9 |

Thisdemonstratestheproblem:somedepartmentsreceiveconsiderablymoreaddressesthan they require, while another department could require more than its assigned subnet can provide.

- IdentifyInefficientAllocations

/26

/27

/28

- Provides62usableaddresses.

- Suitableforadepartmentrequiringupto62 hosts.

- Ifthe department needsonly 20–30 hosts,/26 isinefficient.

- Provides30usableaddresses.

- Suitableforadepartmentrequiringupto30 hosts.

- Ifitneedsonly 10–15hosts,/27wastesaddresses.

- Providesonly14usableaddresses.

- Suitableforsmalldepartments.

- Ifadepartment needsmorethan 14hosts, /28 is insufficient.

Therefore,assigningthesamelargesubnettodepartmentswithdifferent requirements

causesaddresswastage.

# DebuggingtheAddressingPlan

Thesubnetboundariesmustbecheckedcarefully. For /26, the block size is:

256 − 192 = 64

Therefore:

- 192.168.1.0/26 → .0– .63

- 192.168.1.64/26 → .64– .127

- 192.168.1.128/26 → .128– .191

- 192.168.1.192/26 → .192– .255

For /27, the block size is 32. For /28, the block size is 16. Commonallocationmistake

A/27subnetcannotstartatanarbitraryaddresssuchas: 192.168.1.50/27 +

Valid/27networkaddressesare:

192.168.1.0, .32, .64, .96, .128, .160, .192, .224

Similarly,valid/28networkaddressesaremultiplesof16.

# RecommendedVLSMScheme

VLSM(VariableLengthSubnetMasking)allowsdifferentdepartmentstoreceive different-sized subnets according to their actual requirements.

Forexample,supposethedepartments require:

- DepartmentA→50hosts

- DepartmentB→25hosts

- DepartmentC→10hosts

- Department D → 5 hosts Allocatethelargestrequirementfirst. OptimizedAllocation

| Department | Required Hosts | VLSM Subnet | Usable Hosts | Network | Host Range | Broadcast |
| --- | --- | --- | --- | --- | --- | --- |
| A | 50 | /26 | 62 | 192.168.1.0 | .1 – .62 | .63 |
| B | 25 | /27 | 30 | 192.168.1.64 | .65 – .94 | .95 |
| C | 10 | /28 | 14 | 192.168.1.96 | .97 – .110 | .111 |
| D | 5 | /29 | 6 | 192.168.1.112 | .113 – .118 | .119 |

# WhyVLSMIs Better

WithoutVLSM,everydepartmentmightbeassignedasubnetofthesamesize,evenwhen their requirements are different.

WithVLSM:

- Largedepartment→/26

- Mediumdepartment→/27

- Smalldepartment→/28

- Verysmalldepartment→/29

ThisreducesIPaddresswastageandallowstheorganizationtoaccommodatemore departments within the original /24 network.

# FinalAnswer

The existing /26, /27, and /28 allocations are not necessarily wrong, but their efficiency depends on the actual number of hosts required by each department. The main addressing problemoccurswhenadepartmentreceivesasubnetsignificantlylargerthanitsrequirement or a subnet smaller than its requirement. VLSM is the recommended solution because it allocatessubnetsizesaccordingtodepartmentalrequirements.Fortheexamplerequirements of 50, 25, 10, and 5 hosts, the optimized scheme is /26, /27, /28, and /29, respectively.

# ​IPv6AddressingandSubnetDebuggingGiven

  - DeviceA:2001:db8::ff00:42:8329/64

  - DeviceB:2001:db8:0:0:1::1/64

Sincetheprefixlengthis/64,thefirst64bits(4groups)representthenetworkportion.

# ExpandtheIPv6AddressesDeviceA

Given:

2001:db8::ff00:42:8329/64

Expandedform:

# 2001:0db8:0000:0000:ff00:0042:8329

However,anIPv6addressmustcontain8groups,sothecompleteexpandedaddressis:

# 2001:0db8:0000:0000:0000:ff00:0042:8329/64

DeviceB

Given:

2001:db8:0:0:1::1/64

Completeexpandedform:

# 2001:0db8:0000:0000:0001:0000:0000:0001/64

- DeterminetheNetworkPrefixFor /64, take the first four groups.DeviceA

2001:0db8:0000:0000:0000:ff00:0042:8329

Networkprefix:

# 2001:db8:0:0::/64

DeviceB

2001:0db8:0000:0000:0001:0000:0000:0001

Networkprefix:

# 2001:db8:0:0:1::/64

- CheckWhetherBothDevicesAreintheSameSubnet

| Device | IPv6Address | NetworkPrefix |
| --- | --- | --- |
| A | 2001:db8::ff00:42:8329/64 | 2001:db8:0:0::/64 |
| B | 2001:db8:0:0:1::1/64 | 2001:db8:0:0:1::/64 |

Comparethefirstfour groups:

# DeviceA:

2001 : db8 : 0 : 0

# DeviceB:

2001 : db8 : 0 : 0

# Therefore,bothdevicesactuallyhavethesame/64networkprefix. Important Correction

Theearlierconclusionthatthesedevicesareindifferent/64subnetswouldbeincorrect.

The1inDeviceB appearsinthe5thgroup,whichispartoftheinterfaceIDwhentheprefix is /64.

Therefore:

# Bothdevicesbelongtothesamesubnet:

2001:db8:0:0::/64

SotheIPv6addressesthemselvesdonotcontainaprefixmismatch.

# IdentifytheActualConfigurationIssue

Becauseboth devicesare in:

# 2001:db8:0:0::/64

thegivenaddressesarevalid membersofthesamesubnet.

Therefore,if communicationfails,theproblemmust becausedbysomethingelse,such as:

  - IncorrectIPv6subnetprefixconfiguredononedevice

  - IncorrectIPv6addressconfiguration

  - Incorrect/defaultgatewayconfiguration

  - IPv6disabled on one device

  - Incorrectrouterorinterface configuration

  - VLANorLayer-2connectivity problem

  - Firewall/ACLblockingIPv6traffic

  - DuplicateIPv6 address

So,basedonlyonthegivenaddresses,thereisnoaddressing/prefixmismatch.

# OptimizedIPv6Configuration

Wecanuseclearaddressesfromthesame/64subnet.

# DeviceA 2001:db8:0:0::1/64

Device B 2001:db8:0:0::2/64

Both belong to:

NetworkPrefix:2001:db8:0:0::/64

Thisprovidesasimpleandeasy-to-manageaddressingscheme. If a router is present, an appropriate router address could be: 2001:db8:0:0::254/64

| Device | IPv6Address | Prefix | Gateway |
| --- | --- | --- | --- |
| DeviceA | 2001:db8:0:0::1 | /64 | 2001:db8:0:0::254 |
| DeviceB | 2001:db8:0:0::2 | /64 | 2001:db8:0:0::254 |

Thegatewayaddressisonlyanexample;itmustactuallybeconfiguredonthe router.

# NumberofAvailableHostAddresses

IPv6has128bits. For a/64 network:

Hostbits= 128 −64 = 64 bits

Therefore:

Numberofaddresses= 2⁶⁴

# =18,446,744,073,709,551,616 addresses

UnlikeIPv4,IPv6doesnotreserveabroadcastaddress.

# FinalAnswer

| Paramete r | DeviceA | DeviceB |
| --- | --- | --- |
| Given Address | 2001:db8::ff00:42:8329/64 | 2001:db8:0:0:1::1/64 |
| Expanded Address | 2001:0db8:0000:0000:0000:ff00:0042:8329/ 64 | 2001:0db8:0000:0000:0001:0000:0000:0001/ 64 |
| Network Prefix | 2001:db8:0:0::/64 | 2001:db8:0:0::/64 |
| Same Subnet? | Yes | Yes |

# Conclusion

BothdevicesalreadybelongtothesameIPv6/64subnet.Thereisnoprefixmismatchinthe addresses provided. If communication fails, the administrator should investigate the router/gateway, VLAN, firewall, IPv6 interface configuration, or Layer-2 connectivity.

Anoptimizedconfigurationcanuse:

- DeviceA:2001:db8:0:0::1/64

- DeviceB:2001:db8:0:0::2/64

- Gateway:2001:db8:0:0::254/64

- AvailableIPv6addresses:2⁶⁴=18,446,744,073,709,551,616

# ​RoutingTableDebugging Given Routing Table

| DestinationNetwork | NextHop |
| --- | --- |
| 192.168.1.0/24 | 10.0.0.2 |
| 192.168.2.0/24 | 10.0.0.3 |
| DefaultRoute | 10.0.0.1 |

DestinationIP:192.168.2.100

- FindtheMatchingRouteCheck the destination: 192.168.2.100

Therouting table contains:

192.168.2.0/24 → 10.0.0.3

A/24 subnet covers:

192.168.2.0– 192.168.2.255

Therefore:

# 192.168.2.100belongsto192.168.2.0/24.

⬛Matchingroute found.

# Longest-PrefixMatch

Therouterselectsthemostspecificmatching route.

| Route | Match? |
| --- | --- |
| 192.168.1.0/24 | +No |
| 192.168.2.0/24 | ⬛Yes |
| Default0.0.0.0/0 | Generalroute |

Therefore,theselectedrouteis:

192.168.2.0/24 → 10.0.0.3

# NextHop 10.0.0.3

Therouter forwardsthepacketto 10.0.0.3.

# VerifyDestinationSubnet

For /24:

  - Network:192.168.2.0

  - Firsthost:192.168.2.1

  - Lasthost: 192.168.2.254

  - Broadcast:192.168.2.255 Therefore:

# 192.168.2.100is a validhost address.

⬛DestinationIPiscorrectfortheintendedsubnet.

# IdentifyRouting Error

Basedonlyontheroutingtable,thereisnorouting-tableerrorfordestination 192.168.2.100.

Thecorrectrouteisalready present:

192.168.2.0/24 → 10.0.0.3

Ifpacketsstilldonotreachthedestination,possibleproblems include:

  - Next-hoprouter10.0.0.3is unreachable.

  - Router10.0.0.3 doesnot havearouteto 192.168.2.0/24.

  - Incorrectinterfaceconfiguration.

  - Link failure.

  - Firewall/ACLblockingthepackets.

  - Incorrectreturn route.

# OptimizedRoutingTable

Theroutingtableisalreadyreasonablyefficientbecauseitcontainsspecific/24routesanda default route.

| Destination | NextHop | Purpose |
| --- | --- | --- |
| 192.168.1.0/24 | 10.0.0.2 | Network1 |
| 192.168.2.0/24 | 10.0.0.3 | Network2 |
| 0.0.0.0/0 | 10.0.0.1 | Allother networks |

Thespecific routesarepreferredoverthedefaultroutebecauseof longest-prefixmatching.

# RouteUsedWhenNo MatchExists

Ifthe destination doesnot match either:

  - 192.168.1.0/24

  - 192.168.2.0/24

therouterusesthedefault route:

0.0.0.0/0 → 10.0.0.1

Forexample,ifthedestinationis192.168.5.20,thereisnospecificmatchingroute,sothe packet is forwarded to:

# NextHop:10.0.0.1 FinalAnswer

192.168.2.100 belongs to the 192.168.2.0/24 network. Using longest-prefix matching, the router selects192.168.2.0/24 → 10.0.0.3, so the next-hop routeris10.0.0.3.Thedestination IPisvalid.Thereisno errorin thegiven routing tableforthisdestination.If communication

stillfails,theproblemislikelywiththenext-hoprouter,interface,link,firewall,orreturn route.Ifnospecificroute matches, therouter usesthedefault route0.0.0.0/0→ 10.0.0.1.